Active Infrared Counter-Surveillance Eyewear

A Wearable System for Disrupting Non-Consensual Facial Recognition

Senior Design Project Proposal  •  Computer Engineering  •  Team of 2  •  14 June 2026

Team: [Student 1] & [Student 2]      Supervisor: [Advisor]

1.  Problem & Motivation

Facial-recognition cameras are increasingly deployed in public and commercial spaces, often identifying individuals without their consent. Existing countermeasures such as IR-reflective eyewear are passive and rarely validated with measured data. We propose an active wearable device that emits modulated infrared light to disrupt a camera's capture of the wearer's face, paired with a measurement framework that quantifies when and how well it works against real deep-learning recognition systems — turning a privacy gadget into a rigorous engineering study.

2.  Objectives

- Design and build wearable IR-emitter eyewear: a microcontroller driving an infrared LED array through a logic-level MOSFET, powered by a small rechargeable LiPo cell.

- Develop firmware with configurable PWM pulsing (frequency and duty cycle), selectable modes, a control button, low-power sleep, and battery monitoring.

- Build an evaluation framework that quantifies disruption of face detection and recognition using a real camera-plus-software pipeline.

- Characterise effectiveness across distance, lighting, camera type, IR wavelength (850 nm vs 940 nm), and pulse parameters — and honestly analyse the limits (e.g. daytime IR-cut filters).

3.  System Overview

Emitter (hardware): ESP32-C3 controller switching an IR LED array via a logic-level MOSFET from a small LiPo; LEDs embedded in thick eyeglass frames.

Firmware: hardware-PWM pulsing engine to overdrive the LEDs in short bursts, mode/intensity control, deep-sleep power management, and battery sensing.

Evaluation rig: captured frames are run through deep-learning face models (e.g. RetinaFace/MTCNN for detection, ArcFace/dlib for recognition) to measure recognition-failure rate and the shift in face-embedding distance, alongside sensor-overexposure and rolling-shutter banding metrics.

4.  Methodology

We prototype on a breadboard (single LED → full array), integrate the electronics into the frames, then run controlled experiments that sweep the variables above against a real face recognizer. Each configuration is scored on whether the camera can still detect and identify the face, and results are reported as quantitative graphs with a clear analysis of operating conditions and failure modes. As a stretch goal, we frame the disruption as a physical adversarial attack on the recognition network and explore tuning the IR pattern to force misidentification.

5.  Expected Deliverables & Responsibilities

Deliverables: a working prototype, documented firmware, an experimental dataset and results, a final written report, and a live demonstration.

Responsibilities: Member 1 — hardware design, LED/MOSFET circuit, frame integration, firmware; Member 2 — evaluation pipeline, experiments and data analysis, report and demo.

6.  Timeline (6 weeks)

- Week 1 — source parts; breadboard single-LED PWM proof-of-concept (verified on a phone camera).

- Week 2 — full LED array + MOSFET driver; firmware pulsing engine and controls.

- Week 3 — integrate electronics and battery into the eyeglass frames; bench testing.

- Week 4 — build the face-detection/recognition evaluation pipeline and test protocol.

- Week 5 — run experiments across all variables; collect dataset.

- Week 6 — analysis, final report, and demo preparation.

Key reference: Z. Zhou et al., “Invisible Mask: Practical Attacks on Face Recognition with Infrared,” arXiv:1803.04683, 2018 (PWM IR-LED adversarial attack on FaceNet — basis for method and related work).

Safety & ethics: the device operates within infrared photobiological-safety limits (IEC 62471); it saturates camera sensors and does not affect human vision. Intended use is consensual self-protection against non-consensual recognition.